MSD-54

Soft Dome Midrange

| SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| Nominal Power Handling (RMS) |  | P | 200 W |
| Transient Power - 10 ms |  |  | 1,000 W |
| Nominal Impedance |  | Z | 8  |
| Sensitivity 1W/1M |  |  | 90.5 dB |
| Frequency Response |  |  | 500 – 6,500 Hz |
| Resonant Frequency |  | FS | 380 Hz |
| VOICE COIL |  |  |  |
| Voice Coil Diameter |  |  | 54 mm (2.1”) |
| Voice Coil Height |  |  | 6.0 mm (0.23”) |
| Voice Coil Former |  |  | aluminum |
| Voice Coil Wire |  |  | hexagonal shaped aluminum |
| Number of Layers |  |  | 2 |
| DC Resistance |  | RE | 6.3  |
| Voice Coil Inductance @ 1 kHz |  | LBM | 0.19 mH |
| MAGNET SYSTEM |  |  |  |
| Magnet System Type |  |  | neodymium, vented |
| Magnetic Gap Height |  | HE | 4.0 mm (0.16”) |
| Flux Density |  | B | 0.90 T |
| BL Product |  | BXL | 5.2 NA |
| Max. Linear Excursion |  | X |  1 mm |
| OPERATIONAL PARAMETERS |  |  |  |
| Suspension Compliance |  | CMS | - |
| Mechanical Q Factor |  | QMS | - |
| Electrical Q Factor |  | QES | - |
| Total Q Factor |  | Q/T | - |
| Mechanical Resistance |  | RMS | - |
| Moving Mass |  | MMS | 2.2 g |
| Equivalent Cas Air Load |  | VAS | - |
| Cone / Dome Material |  |  | hand treated fabric |
| Effective Piston Area |  | S | 28.0 cm2 |
| Dimensions | Faceplate Diameter Faceplate Thickness Cutout Mounting Depth |  | 87mm (3.43”) 4.0mm (0.16”) 76mm (2.99”) 82mm (3.23”) |
| Net Weight |  |  | 0.30 kg (0.66 lbs) |