MSD-56

2.1” Soft Dome Midrange

| APPLICATION INFORMATION |  |  |  |
| --- | --- | --- | --- |
| Nominal Power Handling (RMS) |  | P | 200 W |
| Transient Power - 10 ms |  |  | 1,000 W |
| Nominal Impedance |  | Z | 8  |
| Sensitivity 1W/1M |  |  | 90.5 dB |
| Frequency Response |  |  | 450 – 6,000 Hz |
| Resonant Frequency |  |  | 340 Hz |
| VOICE COIL |  |  |  |
| Voice Coil Diameter |  |  | 55 mm (2.17” |
| Voice Coil Height |  |  | 7 mm (0.28") |
| Voice Coil Former |  |  | aluminum |
| Voice Coil Wire |  |  | hexagonal shaped aluminum |
| Number of Layers |  |  | 2 |
| DC Resistance |  | RE | 5.2  |
| Voice Coil Inductance @ 1 kHz |  | LBM | 0.19 mH |
| MAGNET SYSTEM |  |  |  |
| Magnet System Type |  |  | ferrite, chambered |
| BL Product |  | BXL | 6.4 NA |
| OPERATIONAL PARAMETERS |  |  |  |
| Mechanical Q Factor |  | QMS | 0.90 |
| Electrical Q Factor |  | QES | 0.66 |
| Total Q Factor |  | Q/T | 0.39 |
| Moving Mass |  | MMS | 3.2 g |
| Cone / Dome Material |  |  | hand treated fabric |
| Effective Piston Area |  | S | 30.0 cm2 |
| Dimensions | Faceplate Diameter Faceplate Thickness Cutout Mounting Depth |  | 140mm (5.51””) 3.5mm (0.14”) 107mm (4.21”) 69mm (2.72”) |
| Net Weight |  |  | 1.3 kg (2.95 lbs) |