MSW-1115

4” Midrange

| SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| Nominal Power Handling (RMS) |  | P | 120 W |
| Transient Power - 10 ms |  |  | 800 W |
| Nominal Impedance |  | Z | 8  |
| Sensitivity 1W/1M |  |  | 87 dB |
| Frequency Response |  |  | 75 – 5,000 Hz |
| Resonant Frequency |  | FS | 78 Hz |
| VOICE COIL |  |  |  |
| Voice Coil Diameter |  |  | 54mm (2.1”) |
| Voice Coil Height |  |  | 10.0 mm (0.39”) |
| Voice Coil Former |  |  | aluminum |
| Voice Coil Wire |  |  | hexagonal shaped aluminum |
| Number of Layers |  |  | 2 |
| DC Resistance |  | RE | 5.6  |
| Voice Coil Inductance @ 1 kHz |  | LBM | 0.42 mH |
| MAGNET SYSTEM |  |  |  |
| Magnet System Type |  |  | neodymium, vented |
| Magnetic Gap Height |  | HE | 6.0 mm (0.24”) |
| Flux Density |  | B | 0.88 T |
| BL Product |  | BXL | 7.06 NA |
| Max. Linear Excursion |  | X | 2.0 mm (0.08”) |
| OPERATIONAL PARAMETERS |  |  |  |
| Suspension Compliance |  | CMS | 529 M / newton |
| Mechanical Q Factor |  | QMS | 2.29 |
| Electrical Q Factor |  | QES | 0.44 |
| Total Q Factor |  | Q/T | 0.37 |
| Mechanical Resistance |  | RMS | 1.347 Kg S-1 |
| Moving Mass |  | MMS | 6.30 g |
| Equivalent Cas Air Load |  | VAS | 2.76 L |
| Cone / Dome Material |  |  | damped polymer composite |
| Effective Piston Area |  | S | 61 cm2 |
| Dimensions | Diameter Cutout Height Mounting Depth |  | 118mm (4.65”) 98mm (3.86”) 58mm (2.28”) 54mm (2.13”) |
| Net Weight |  |  | 0.50 kg (1.1 lbs) |