MSW-1167

6” Poly Cone Midbass

| SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| Nominal Power Handling (RMS) |  | P | 150 W |
| Transient Power - 10 ms |  |  | 1,000 W |
| Nominal Impedance |  | Z | 8  |
| Sensitivity 1W/1M |  |  | 88 dB |
| Frequency Response |  |  | 45 – 4,500 Hz |
| Resonant Frequency |  | FS | 44 Hz |
| VOICE COIL |  |  |  |
| Voice Coil Diameter |  |  | 75mm (3”) |
| Voice Coil Height |  |  | 14.5 mm (0.57”) |
| Voice Coil Former |  |  | aluminum |
| Voice Coil Wire |  |  | hexagonal shaped aluminum |
| Number of Layers |  |  | 2 |
| DC Resistance |  | RE | 6.6  |
| Voice Coil Inductance @ 1 kHz |  | LBM | 0.08 mH |
| MAGNET SYSTEM |  |  |  |
| Magnet System Type |  |  | neodymium, vented |
| Magnetic Gap Height |  | HE | 6 mm (0.24”) |
| Magnetic Gap Width |  |  | 1.35mm (0.05”) |
| Flux Density |  | B | 0.82 T |
| BL Product |  | BXL | 7.80 NA |
| Max. Linear Excursion |  | X | 4.25 mm (0.17”) |
| OPERATIONAL PARAMETERS |  |  |  |
| Suspension Compliance |  | CMS | 945 M / newton |
| Mechanical Q Factor |  | QMS | 1.99 |
| Electrical Q Factor |  | QES | 0.43 |
| Total Q Factor |  | Q/T | 0.35 |
| Moving Mass |  | MMS | 14 g |
| Equivalent Cas Air Load |  | VAS | 18.80 liters |
| Cone / Dome Material |  |  | damped polymer composite |
| Effective Piston Area |  | S | 119 cm2 |
| Dimensions | Diameter Cutout Height Mounting Depth |  | 160mm (6.30”) 134mm (5.28”) 68.5mm (2.70”) 62mm (2.44”) |
| Net Weight |  |  | 1.1 kg (2.4 lbs) |