MSW-12.1

12” High-Power Subwoofer

| SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| Power Handling (RMS) |  | P | 400 W |
| Transient Power Handling |  |  | 1,500 watts |
| Sensitivity SPL @ 1watt |  | SPL0 | 90.8 dB |
| Nominal Impedance |  |  | 8 W |
| Resonant Frequency |  | Fs | 23.3 Hz |
| VOICE COIL |  |  |  |
| Voice Coil Height |  |  | 30mm |
| DC Resistance |  | Revc | 6.2 W |
| Zmax @ F0 |  | Z0 | 217.0 W |
| MAGNET SYSTEM |  |  |  |
| Flux x Length |  | BL | 21.6 TM |
| Maximum Linear Excursion |  | Xmax | 10mm |
| Ref Efficiency |  | N0 | 0.76 % |
| OPERATIONAL PARAMETERS |  |  |  |
| Compliance |  | Cms | 377.3 mM/N |
| Mechanical Q Factor |  | Qms | 8.21 |
| Electrical Q Factor |  | Qes | 0.24 |
| Total Q Factor |  | Qts | 0.23 |
| Moving Mass |  | Mms | 123.4 grams |
| Diaphragm Mass |  | Mmd | 116.4 grams |
| Equivalent Cas Air Load |  | VAS | 150.5 Litres |
| Cone Material |  |  | polypropylene |
| Surround Material |  |  | santopreneÔ |
| Piston Area |  | Sd | 0.05 M2 |
| Dimensions | Diameter Cutout Height Mounting Depth |  | 315mm (12.4”) 283mm (11.1”) 156mm (6.1”) 140mm (5.5”) |
| Net Weight |  |  | 6.36 kg (14 lbs) |